Fundamentals of Control Systems

Elec 372

Lab Experiment #2

Andre Hei Wang Law

4017 5600

Section UJ-X

TA: Saba Sanami

TA Email: sabasanami272@gmail.com

Professor: Amir Aghdam

Performed on February 13, 2024

Due on March 5, 2024

“I certify that this submission is my original work and meets the Faculty’s Expectations of

Originality.”

Andre Hei Wang Law

4017 5600

05/03/2024

1) Objectives

For the second lab experiment of the course Elec 372, students are tasked to perform system identification on a mechanical first-order system to determine its inertia, damping and gain (J, B, K) parameters. The experiment will involve open-loop and closed-loop response on a DC motor. They will also perform calculations on the system model and verifying the model using MATLAB. As such, these theoretical predictions and the experimental results will allow for students to compare data which deepens their understanding on the fundamentals of control systems.

2) Theory

Figure 2.1

The above figure represents a ECP 220 system block diagram of a closed-loop configuration. A PI+V controller configuration is considered as the “default” configuration as it allows for a good control of closed-loop system parameters.

Figure 2.2

The figure 2.2 demonstrates the rotation inertia ratio between disks. The letter of the above figure represents:

D: The drive, inertia specified by the manufacturer is 4(10)-4kgm2

L: The load, inertia specified by the manufacturer is 65(10)-4kgm2

SR: The SR disks, inertia specified by the manufacturer is 78(10)-6kgm2

w: the additional inertia of the weights fixed to the driver and the load disks

3) Tasks / Results / Discussions

3.1) EVALUATION OF J AND K USING OPEN-LOOP VELOCITY OUTPUT

Figure 3.1.1

The left axis is Encoder #1 velocity and the x axis is time in seconds. Notice that the large linear increase of velocity on the y axis at the start represents a constant acceleration. Around time 0.5 seconds, we reach the maximum and the velocity decreases. This occurs because the step reverse and the “overload” circuit disable the system at this point.

Figure 3.1.2

This is similar is obtain by performing the exact same procedure as the one before it, however we added an additional weight to the system. Notice how the shape of the graph of figure 3.2 is very similar to figure 3.1. Both increases linearly and both stops around 0.5 seconds. However, a major difference is the peak. For the non-weighted system, the peak reached around 116000, while the weighted system could only reach 106000. This means that had a higher velocity or acceleration rate compared to the weighted system.

Using Equation (4.4), determine the value of J and compare the value obtained with the calculated nominal value given in sections (4.2.1) and (4.2.2) and find the % error.

J=a2a1-a2ΔJ kgm2

a1=y2-y2x2-x2=98000-200000.4-0.085=247619.05 counts/sec2

a2=y2-y2x2-x2=80000-400000.37-0.18=210526.32 counts/sec2

ΔJ=0.000494

The experimental value of J is calculated as such:

J=a2a1-a2ΔJ=210526.32247619.05-210526.32*0.000494=2.8*10-3

The % error is calculated as such:

% error=Actual Value-Experimental ValueExperimental Value=4.23*10-3-2.8*10-32.8*10-3=0.51

The percent error is around 51%. It is unavoidable that we obtain a high percent error since the values of y and x used to calculated a1 and a2 are rough estimates.

Using the determined value of J, find KaKtKe using Equation (4.3). Multiply this value by KsKc to obtain K (see section 4.2.1) for the equipment at your lab station.

Ja1=2KaKtKe

KaKtKe=Ja12=2.8*10-3*247619.052=346.67

K=KsKcKeKaKt=346.67*0.009766=3.39

The value of K obtained by using the determined value of J is 3.39.

3.2) EVALUATION OF J AND K USING OPEN-LOOP POSITION OUTPUT

Figure 3.2.1

The left axis is encoder #1 position while CE is the right axis with both in relation to time on the x axis. This can be visually seen with the red line (position) and the yellow line (CE).

Determine the slope S of the line (in Counts/sec) and the time-axis intercept τ (sec).

S=y2-y2x2-x2=700000-3000006.8-4= 142857.14 count/sec

Convert from counts to radians:

S(rad/sec)=S(count/sec)Ke=142857.14 2546.5=56.099 radian/sec

B=0.1S(radians/sec)=1.7825*10-3

From the displayed time-axis intercept, determine the time-constant τ = J/B from x-axis intercept.

Based on figure 3.2.1, the intercept is at 2 seconds.

Determine J:

J=τ*B=2*1.7825*10-3=3.565*10-3

As such, the experimental value of J is 3.565*10-3 and B is 1.7825*10-3.

Compare with the determined value of 4.3.1 and obtain the average value:

The value of J in 4.3.1 is 2.8*10-3.

The value earlier is J is 3.565*10-3.

As such, the average value of J is 3.1825*10-3.

Percentage Error:

% error=Actual Value-Experimental ValueExperimental Value=4.23*10-3-3.1825*10-33.1825*10-3=0.33

The percentage error is around 33%.

Record the average value of J, and the values of B and K obtained, these values are to be used in

subsequent lab work.

Average of J: 3.1825*10-3 kgm2

B: 1.7825*10-3 Nm/radian

K: 3.39 Nm/radian

3.3) SYSTEM IDENTIFICATION VIA THE OPEN-LOOP RESPONSE TO A SINE SWEEP INPUT

Figure 3.3.1

The above figure has CE displayed on the left axis and encoder #1 velocity on the right axis with both dependent on times in seconds. This is an open-loop system with sweep input of 0.1 to 10Hz and 0.5 volt with sweep time of 20 seconds.

Figure 3.3.2

The above figure represents the MATLAB model obtained when running the MATLAB code found in the appendix. We can observe that figure 3.3.1 and 3.3.2 are the exact same. The left MATLAB figure is the red line CE, while the right figure is the yellow velocity.

3.4) SYSTEM IDENTIFICATION VIA CLOSED-LOOP SYSTEM RESPONSE

Figure 3.4.1

The commanded position and encoder #1 position are both on the left axis. The algorithm input is Kp = 1, Kd = 0 and Ki = 0.

Figure 3.4.2

The system was running exactly the same except that we changed the algorithm input to Kp = 1, Kd = 0.01 and Ki = 0.

Their MATLAB simulation counterparts are as follows:

Figure 3.4.3

Figure 4.3.4

It can be observed that their MATLAB counterparts are very similar, thus proving that our experimental and theoretical systems are behaving accordingly. Given these sounds results, we can calculate and find the first-order transfer functions.

Calculate system the open-loop transfer function (OLTF) with the determined values of K, J and B from open-loop tests in section 4.3.2 and 4.3.3.

OLFT=KsB+Js=3.39s1.7825*10-3+3.1825*10-3*s

Obtain the OLTF in MATLAB by the data of the closed-loop test., then calculate K, B and J.

tfd2=6623s2+28.07s+6611=KsB+Js

Given this equation, we can find K = 6623, B = 28.07 and J = 1.

Compare these models

For this experiment, we have obtained models from the lab equipment as well as the simulation done through MATLAB. It can be observed from those figures that both types of models are similar. While we might encounter slight differences when observing specific data points, the general tendencies are similar.

Write a brief summary about how the lab equipment might facilitate a better understanding of control systems

Theory learned during lectures can only go so far. The best method to encapsulates theory is through practice. As such, by having students perform the experiment by themselves, they are able to grasp the topic and understand it better. In other words, lab equipment allows for experimentations which in turn facilitate a better understanding. In the end, this approach bridges the theory with practical application as it provides hands-on experience.

4) Questions

None

5) Conclusions

In conclusion, the students performed experiments on system identification and control system analysis of an open-loop and closed-loop response systems. We observed the effect of inertia, damping coefficient and gain. Then, by performing both the experiment with the lab equipment and a simulation through MATLAB, students are able to compare plots and analyse the effect due to changes in parameters.

6) Appendix

MATLAB Code used for this lab:

time = data(:,2);
y = data(:,4);
u = data(:,6);
dy = diff(y);
dy(end + 1) = dy(end);
zf = iddata(dy,u,0.00884);
tfl = tfest(zf,1,0);
plot(time, y);
plot(time, u);

time = data(:,2);
y = data(:,4);
u = data(:,6);
dy = diff(y);
dy(end + 1) = dy(end);
zf = iddata(dy,u,0.00884);
tfl = tfest(zf,1,0);
plot(time, y);
plot(time, u);

time = data(:,2);
y = data(:,4);
u = data(:,3);
zf = iddata(y,u,0.00884);
Tfd2 = tfest(zf,2,0);

plot(time, y);
hold on;
plot(time, u);

Sample example of gather raw data used in MATLAB

Sample   Time     Commanded Pos   Encoder 1 Pos   Encoder 2 Pos   Control Effort

[    0    0.000            1000               0               0          0.0000;
     1    0.022            1000             125              12          4.8840;
     2    0.044            1000             445              95          4.0611;
     3    0.066            1000             942             229         -1.3022;
     4    0.089            1000            1354             355         -4.7595;
     5    0.111            1000            1574             409         -4.8840;
     6    0.133            1000            1537             405         -4.8565;
     7    0.155            1000            1278             334         -1.5201;
     8    0.177            1000             929             234          1.9359;
     9    0.199            1000             698             160          3.6044;
    10    0.221            1000             663             152          3.1746;
    11    0.243            1000             829             198          0.9225;
    12    0.266            1000            1053             262         -1.3352;
    13    0.288            1000            1204             307         -2.4066;
    14    0.310            1000            1222             315         -2.0977;
    15    0.332            1000            1121             286         -0.7070;
    16    0.354            1000             976             246          0.7552;
    17    0.376            1000             875             217          1.5079;
    18    0.398            1000             860             211          1.3272;
    19    0.421            1000             926             230          0.4164;
    20    0.443            1000            1018             255         -0.5055;
    21    0.465            1000            1080             272         -0.9542;
    22    0.487            1000            1088             275         -0.8474;
    23    0.509            1000            1046             263         -0.2521;
    24    0.531            1000             987             248          0.3358;
    25    0.553            1000             948             237          0.6215;
    26    0.576            1000             943             235          0.5714;
    27    0.598            1000             971             242          0.1551;
    28    0.620            1000            1008             252         -0.2125;
    29    0.642            1000            1032             259         -0.3822;
    30    0.664            1000            1034             260         -0.3462;
    31    0.686            1000            1015             255         -0.0714;
    32    0.708            1000             992             249          0.1593;
    33    0.730            1000             977             245          0.2637;
    34    0.753            1000             976             244          0.2393;
    35    0.775            1000             988             247          0.0562;
    36    0.797            1000            1003             251         -0.0836;
    37    0.819            1000            1012             253         -0.1477;
    38    0.841            1000            1013             254         -0.1313;
    39    0.863            1000            1006             253         -0.0220;
    40    0.885            1000             996             250          0.0659;
    41    0.908            1000             991             248          0.1136;
    42    0.930            1000             990             248          0.1026;
    43    0.952            1000             994             248          0.0379;
    44    0.974            1000            1000             250         -0.0214;
    45    0.996            1000            1004             251         -0.0476;
    46    1.018            1000            1004             251         -0.0433;
    47    1.040            1000            1003             251          0.0024;
    48    1.063            1000            1000             251          0.0147;
    49    1.085            1000             998             251          0.0311;
    50    1.107            1000             997             251          0.0385;
    51    1.129            1000             997             251          0.0342;
    52    1.151            1000             998             251          0.0195;
    53    1.173            1000             999             251          0.0012;
    54    1.195            1000            1000             251         -0.0079;
    55    1.217            1000            1000             251         -0.0043;
    56    1.240            1000            1000             251         -0.0043;
    57    1.262            1000            1000             251         -0.0043;
    58    1.284            1000            1000             251         -0.0043;
    59    1.306            1000            1000             251         -0.0043;
    60    1.328            1000            1000             251         -0.0043;
    61    1.350            1000            1000             251         -0.0043;
    62    1.372            1000            1000             251         -0.0043;
    63    1.395            1000            1000             251         -0.0043;
    64    1.417            1000            1000             251         -0.0043;
    65    1.439            1000            1000             251         -0.0043;
    66    1.461            1000            1000             251         -0.0043;
    67    1.483            1000            1000             251         -0.0043;
    68    1.505            1000            1000             251         -0.0043;
    69    1.527            1000            1000             251         -0.0043;
    70    1.549            1000            1000             251         -0.0043;
    71    1.572            1000            1000             251         -0.0043;
    72    1.594            1000            1000             251         -0.0043;
    73    1.616            1000            1000             251         -0.0043;
    74    1.638            1000            1000             251         -0.0043;
    75    1.660            1000            1000             251         -0.0043;
    76    1.682            1000            1000             251         -0.0043;
    77    1.704            1000            1000             251         -0.0043;
    78    1.727            1000            1000             251         -0.0043;
    79    1.749            1000            1000             251         -0.0043;
    80    1.771            1000            1000             251         -0.0043;
    81    1.793            1000            1000             251         -0.0043;
    82    1.815            1000            1000             251         -0.0043;
    83    1.837            1000            1000             251         -0.0043;
    84    1.859            1000            1000             251         -0.0043;
    85    1.882            1000            1000             251         -0.0043;
    86    1.904            1000            1000             251         -0.0043;
    87    1.926            1000            1000             251         -0.0043;
    88    1.948            1000            1000             251         -0.0043;
    89    1.970            1000            1000             251         -0.0043;
    90    1.992            1000            1000             251         -0.0043;
    91    2.014               0             958             250         -4.8840;
    92    2.036               0             724             199         -4.8840;
    93    2.059               0             286              86         -0.9280;
    94    2.081               0            -182             -54          3.3529;
    95    2.103               0            -494            -143          4.8840;
    96    2.125               0            -583            -163          4.8840;
    97    2.147               0            -423            -124          3.3303;
    98    2.169               0             -98             -30         -0.3254;
    99    2.191               0             211              62         -3.0531;
   100    2.214               0             347             102         -3.6783;
   101    2.236               0             269              81         -2.0940;
   102    2.258               0              58              20          0.2411;
   103    2.280               0            -141             -36          2.0238;
   104    2.302               0            -226             -63          2.3748;
   105    2.324               0            -184             -51          1.4927;
   106    2.346               0             -52             -15         -0.0183;
   107    2.368               0              81              22         -1.2289;
   108    2.391               0             143              40         -1.5263;
   109    2.413               0             117              34         -0.9420;
   110    2.435               0              32              10          0.0293;
   111    2.457               0             -51             -12          0.7668;
   112    2.479               0             -87             -22          0.9274;
   113    2.501               0             -68             -18          0.5281;
   114    2.523               0             -12              -3         -0.1007;
   115    2.546               0              40              12         -0.5574;
   116    2.568               0              62              18         -0.6380;
   117    2.590               0              47              15         -0.3700;
   118    2.612               0              11               5          0.0421;
   119    2.634               0             -21              -4          0.3053;
   120    2.656               0             -33              -7          0.3407;
   121    2.678               0             -22              -6          0.1538;
   122    2.701               0               1               1         -0.1062;
   123    2.723               0              21               6         -0.2741;
   124    2.745               0              27               8         -0.2680;
   125    2.767               0              20               8         -0.1526;
   126    2.789               0               5               3          0.0110;
   127    2.811               0              -7               0          0.1068;
   128    2.833               0             -11              -1          0.1026;
   129    2.855               0              -6              -1          0.0354;
   130    2.878               0               4               1         -0.0769;
   131    2.900               0              11               4         -0.1386;
   132    2.922               0              13               5         -0.1313;
   133    2.944               0              11               5         -0.0958;
   134    2.966               0               5               4         -0.0281;
   135    2.988               0               1               2         -0.0049;
   136    3.010               0               1               2         -0.0049;
   137    3.033               0               2               2         -0.0531;
   138    3.055               0               6               2         -0.0672;
   139    3.077               0               7               2         -0.0726;
   140    3.099               0               7               2         -0.0726;
   141    3.121               0               7               2         -0.0726;
   142    3.143               0               5               2         -0.0379;
   143    3.165               0               4               2         -0.0324;
   144    3.188               0               4               2         -0.0342;
   145    3.210               0               5               2         -0.0824;
   146    3.232               0               6               2         -0.0653;
   147    3.254               0               6               2         -0.0672;
   148    3.276               0               6               2         -0.0629;
   149    3.298               0               5               2         -0.0488;
   150    3.320               0               5               2         -0.0488;
   151    3.342               0               5               2         -0.0488;
   152    3.365               0               5               2         -0.0488;
   153    3.387               0               5               2         -0.0488;
   154    3.409               0               5               2         -0.0488;
   155    3.431               0               5               2         -0.0488;
   156    3.453               0               5               2         -0.0488;
   157    3.475               0               5               2         -0.0488;
   158    3.497               0               5               2         -0.0488;
   159    3.520               0               5               2         -0.0488;
   160    3.542               0               5               2         -0.0488;
   161    3.564               0               5               2         -0.0488;
   162    3.586               0               5               2         -0.0488;
   163    3.608               0               5               2         -0.0488;
   164    3.630               0               5               2         -0.0488;
   165    3.652               0               5               2         -0.0488;
   166    3.674               0               5               2         -0.0488;
   167    3.697               0               5               2         -0.0488;
   168    3.719               0               5               2         -0.0488;
   169    3.741               0               5               2         -0.0488;
   170    3.763               0               5               2         -0.0488;
   171    3.785               0               5               2         -0.0488;
   172    3.807               0               5               2         -0.0488;
   173    3.829               0               5               2         -0.0488;
   174    3.852               0               5               2         -0.0488;
   175    3.874               0               5               2         -0.0488;
   176    3.896               0               5               2         -0.0488;
   177    3.918               0               5               2         -0.0488;
   178    3.940               0               5               2         -0.0488;
   179    3.962               0               5               2         -0.0488]